| Sorts | Type | Parameters |
| --- | --- | --- |
| System | Operating system | Embedded Linux |
|  | Online visitor | Supports 4 visitors at the same time |
|  | Image Sensor | 1/4.5 inch 1M CMOS image sensor |
|  | Minimum illumination | 0.5 Lux(Color mode), 0.1 Lux(B&W mode) |
|  | Lens/view angle |  |
|  | Night vision | Dual filter automatic switching, white light, illumination distance 15-20 meters |
|  | Compression standard | H.264 main profile/Motion-JPEG/JPEG |
|  | Data rate | Main stream：1080p(1920×1080)@30fps/1296p(2304×1296)@15fps Substream：360p(640×360)@15fps |
|  | Bit rate/maximum frame rate | 128～4096kbps/30fps |
|  | Image adjustment | Adjustable brightness and contrast |
| Audio | Input | Built-in -38dB microphone |
|  | Output | External 8 Ω 1W high sound quality |
|  | Sampling frequency/bit width | 8KHz/16bit |
|  | Compression standard/bit Rate | G.711/64kbps |
| Network | Network interface | No |
|  | Network protocol | TCP/IP,HTTP,TCP,UDP,DHCP,DNS,NTP,RTSP,P2P etc. |
|  | Wireless network | IEEE802.11b/g/n; Support AP mode |
|  | Wireless frequency | 2.4~2.4835GHz |
|  | Wireless security encryption | 64/128-bit WEP/WPA-PSK/WPA2-PSK data encryption |
| Pan&Tilt | Rotation angle | Horizontal 355°/Vertical 60° |
|  | Preset position | Supports 16 presets (mobile app only supports 5) |
| Storage | Storage functions | Supports T-Flash card(max supports 256GB); Cloud Storage;NVR;NAS |
| Alarm | Alarm detection | Supports motion detection, humanoid detection, human tracking, Voice and flash alarm, illumination function,timing white light ON/OFF |
| Physical indicators | Rated voltage | DC5V |
|  | Power consumption |  |
|  | Operating condition | Temperature: -10～50℃; Humidity: ＜90% |
|  | Weight | (Note: in kind prevail） |
|  | Package size | (Note: in kind prevail） |