| System | Operation system | Embedded Linux OS |
| --- | --- | --- |
|  | Online visitor | Support 4 visitors at the same time |
| Image Capture | Image sensor | HI3516E V300 1/3 inch Progressive Scan CMOS |
|  | Minimum Illumination | 0.5Lux（Color mode）,0.1Lux（B&W mode） |
|  | Lens | 4.0mm |
|  | View Angle | 101°（diagonal） |
|  | Night vision | The dual filter switches automatically，2pcs 28mil patch IR LED IR irradiation distance:5~10m |
| Video | Encode format | H.264 main profile/H.264+ main profile/Motion-JPEG/JPEG |
|  | Resolution | 2560x1440@30pfs/640*360@15pfs |
|  | Bit rate/Frame rate/Display frequency | 128～4096kbps/1～30pfs/60Hz、50Hz |
|  | Image adjustment | Brightness and contrast are adjustable |
| Audio | Input | Built-in -38 dB microphone |
|  | Output | Built-in speaker(8Ω1W) |
|  | Sampling frequency / width | 8KHz/16bit |
|  | Compression standard | ADPCM/32kbps |
| Network | Ethernet | RJ-45(10 / 100Base-T) |
|  | Protocols | TCP/IP,HTTP,TCP,UDP,DHCP,DNS,NTP,RTSP,P2P etc |
|  | Wireless LAN | IEEE802.11b/g/n |
|  | Wireless LAN Frequency | 2.4~ 2.4835GHz; 5G |
|  | Wireless LAN security | 64/128-bit WEP/WPA-PSK /WPA2-PSK data encryption |
| Pan-tilt | Pan-tilt | Pan：355°, tilt：110° |
|  | Preset position | PC APP：support 16 positions, Phone APP:support 5 positions |
| Memory | Socket | TF Push-push socket,Maximum support 256GB; Cloud Storage |
| Alarm | Alarm Trigger | Motion detection, Humanoid detection, Humanoid tracking, Crying detection, Smoke detector alarm detection, absence monitoring, Electronic fence |
| Physical Environmental | Rated voltage | DC 5V±5% |
|  | Power consumption | Related power: 2.8W(IR LED on)，Maximum power：4.3W（Pan&Tilt on） |
|  | Operating condition | Temperature:-10～50℃,humidity:＜90% |
|  | Weight | Net: 185 g Gross：450 g(Note: in kind prevail) |
|  | Package size | 185x125x92 mm(L×W×H) |