| Sorts | Type | Parameters |
| --- | --- | --- |
| Collection | Operating system | Embedded Linux |
|  | Online visitor | Supports 4 visitors at the same time |
|  | CPU/Image sensor | 1/4 inch 720p progressive scan CMOS sensor |
|  | Minimum illumination | 0.5 Lux(Color mode), 0.1 Lux(B&W mode) |
|  | Lens | 4mm |
|  | Night Vision | IR-CUT with auto-switching, dual light source, irradiation distance 15-30m |
|  | Compression standard | H.264 main profile/Motion-JPEG/JPEG |
|  | Data rate | Mainstream：(1280×720)@25fps；Substream：360p(640×360)@15fps |
|  | Bit rate/maximum frame rate | 128～4096kbps/25fps |
|  | Image adjustment | Adjustable brightness and contrast |
| Audio | Input | Built-in microphone |
|  | Output | Integrated speaker |
|  | Sampling frequency/bit width | 8KHz/16bit |
|  | Compression standard/Bit Rate | G.711/64kbps |
| Network | Network interface | 10Mbps/100Mbps adaptive /RJ45 interface |
|  | Network protocol | TCP/IP,HTTP,TCP,UDP,DHCP,DNS,NTP,RTSP,P2P etc. |
|  | Wireless network |  |
|  | Wireless frequency | 2.4G |
|  | Wireless security encryption |  |
| Pan&Tilt | Rotation angle | Horizontal 350 °/Vertical 90 °, can be installed with 360 ° rotation |
|  | Preset position | Supports 16 presets (mobile app only supports 5) |
| Storage | Storage functions | Supports T-Flash card(max supports 256GB); Cloud Storage; NVR；NAS |
| Alarm | Alarm detection | Supports motion detection, humanoid detection, human tracking, voice prompts and flashing alarm, irradiation function timed activation and shutdown of white light. |
| Physical indicators | Rated voltage | DC12V |
|  | Power consumption | Rated power: 4.5W (infrared light on)/maximum power: 11W (PTZ on) |
|  | Operating condition | Temperature: -10～50℃; Humidity: ＜90% |
|  | Weight | Note: in kind prevail |
|  | Package size | Note: in kind prevail |