| Sorts | Type | Parameters |
| --- | --- | --- |
| System | Operating system | Embedded Linux |
|  | Online visitor | Supports 4 visitors at the same time |
|  | Image Sensor | 1/3 inch 1440p progressive scan CMOS sensor |
|  | Minimum illumination | 0.5 Lux(Color mode), 0.1 Lux(B&W mode) |
|  | Lens | 4mm-6mm |
|  | Night vision | Dual filter automatic switching, dual light source, illumination distance 15-30 meters. |
|  | Compression standard | H.264 main profile/H.264+ main profile/Motion-JPEG/JPEG |
|  | Data rate | Main stream：1440p(2560×1440)@25fps, Substream：360p(640×360)@15fps |
|  | Bit rate/maximum frame rate | 128～4096kbps/25fps |
|  | Image adjustment | Adjustable brightness and contrast |
| Audio | Input | Built-in -38dB microphone |
|  | Output | Built-in 8 Ω 1W speaker |
|  | Sampling frequency/bit width | 8KHz/16bit |
|  | Compression standard/bit Rate | G.711/64kbps |
| Network | Network interface | 10Mbps/100Mbps adaptive/RJ45 interface |
|  | Network protocol | TCP/IP, HTTP, TCP, UDP, DHCP, DNS, NTP, RTSP, P2P etc. |
|  | Wireless network |  |
|  | Wireless frequency | 2.4G |
|  | Wireless security encryption |  |
| Pan&Tilt | Rotation angle | Horizontal 350°/Vertical 90° |
|  | Preset position | Supports 16 presets (mobile app only supports 5) |
| Storage | Storage | Supports T-Flash card(max supports 256GB); Cloud Storage;NVR;NAS; |
| Alarm | Alarm detection | Supports motion detection, humanoid detection, human tracking, Voice and flash alarm, illumination function,timing white light ON/OFF |
| Physical indicators | Rated voltage | DC12V |
|  | Power consumption | Rated power: 4.5W (infrared light on)/maximum power: 11W (pan&tilt on) |
|  | Operating condition | Temperature: -10～50℃; Humidity: ＜90% |
|  | Weight | (Note: in kind prevail） |
|  | Package size | (Note: in kind prevail） |