|  | Features | ✮ 2K ultra HD. WiFi or network cable connection ✮ Night vision range 5-10 meters ✮ Dual lens, free choice of far and near focus, stepless 5x hybrid zoom ✮ Two-way talk ✮Intelligent human detection and alarm push |
| --- | --- | --- |
| Sorts | Type | Parameters |
| System | Operating system | Embedded Linux |
|  | Online visitor | Supports 4 visitors at the same time |
|  | Image sensor | 1/3 inch CMOS sensor |
|  | Minimum illumination | 0.5 Lux(Color mode), 0.1 Lux(B&W mode) |
|  | Lens | 4mm short focus lens+ 12mm long focus lens, angle of view100°/30° |
|  | Night vision | Automatic switch dual filters, 8 IR lights, night vision range 5-10 meters |
|  | Compression standard | H.264 main profile/Motion-JPEG/JPEG |
|  | Data rate | Mainstream：1296p(2304×1296)@25fps Substream：360p(640×360)@25fps |
|  | Bit rate /maximum frame rate | 128～4096kbps/25fps |
|  | Image adjustment | Adjustable brightness and contrast |
| Audio | Input | Built-in -38dB microphone |
|  | Output | Built-in 8 Ω 1W speaker |
|  | Sampling frequency/bit width | 8KHz/16bit |
|  | Compression standard/bit Rate | G.711/64kbps |
| Network | Network interface | 10Mbps/100Mbps adaptive/RJ45 interface |
|  | Network protocol | TCP/IP,HTTP,TCP,UDP,DHCP,DNS,NTP,RTSP,P2P etc. |
|  | Wireless network | IEEE802.11b/g/n；Support AP mode. |
|  | Wireless frequency | 2.4GHz |
|  | Wireless security encryption | 64/128-bit WEP/WPA-PSK/WPA2-PSK data encryption |
| Pan&Tilt | Rotation angle | Horizontal 348 °/Vertical 70 ° (visible angle 360 °) |
|  | Preset position | Supports 16 presets (mobile app only supports 5) |
| Storage | Storage | Supports TF card(max 256GB); Cloud Storage;NVR;NAS; |
| Alarm | Alarm detection | Support for motion detection, human detection, humanoid framing,human tracking |
| Physical Indicators | Rated voltage | DC 5V±5% |
|  | Power consumption | Rated power: 2.9W (infrared light on)/maximum power: 6.6W (pan &tilt on) |
|  | Operating condition | Temperature: -10～50℃; Humidity: ＜90% |
|  | Weight | N.W.:200 g, G.W.:360 g (Note: in kind prevail） |
|  | Package size | 170x150x108mm(LxWxH) |