id h

| Category | Type | Parameters |
| --- | --- | --- |
| System | Operating system | linux |
|  | Online vistors | 4 users watch online at the same time |
| Collection | Processor | T31 |
|  | Image Sensor | GC2083 , 1/ 3 CMOS (Pixel selectable, PCB compatible) |
|  | Minimum illumination | 0 .3 ~ 0.5Lux (Color mode), 0Lux (B&W mode) |
|  | Lens | 4G lens, IR CUT integrated, standard lens diameter 10mm |
|  | Viewing angle | D135 ° diagonal |
| Video | Night vision | Black and white night vision, photosensitive automatic recognition of light switching, 6pcs 940nm non-luminous patch infrared lights,illumination distance : 3 to 5 meters |
|  | Compression standard | H.264/MJPEF/JPEG |
|  | Multi- stream | Main stream: 1296P/ 15fps Sub-stream: 360P/ 15fps |
|  | Bit rate | CBR/ VBR two rate control modes, and the output rate range is 128 ~ 4096kbps |
|  | Image adjustment | Brightness, contrast, tilt, pan adjustable, OSD display |
| Audio | Input | Built- in - 38dB microphone |
|  | Output | Built- in 8Ω1W speaker |
|  | Sampling frequency/ bit | 8KHz/ 16bit |
|  | Compression standard/ code rate | G711A |
| AI | Humanoid detection | Humanoid detection, filtering invalid video content |
|  | Smart notification sound | Different devices push sound difference, know exactly which device is triggered |
|  | Smart alarm sound | Set different equipment alarm sounds according to different scenarios |
| Network | Wireless connection | Scan the QR code of the device to add/Scan the QR code of the App |
|  | China Cat 1 | FDD:B1/ B3/B5/B8 TDD:B34/B38/B39/B40/B41 |
|  | Overseas Cat4 | LTE-FDD: B1/B3/B7/B8/B20 LTE-TDD: B40/B41 |
| Key | Toggle button 1 | ON/ OFF |
|  | Toggle button 2 | Mode switching ( 4 G networking mode and DV local recording mode switching) |
|  | Reset button | Long press to restore factory settings |
| Storage | External TF card | Support T-Flash card (up to 256GB) |
|  | Cloud Storage | Free Cloud Storage |
| Alarm | Alarm detection | Support Humanoid & Motion detection, PIR detection |
| Physical indicators | Rated voltage | DC5V± 5% |
|  | Powered by | Powered by 2000 mAh battery |
|  | Power consumption | Sleep standby current 5mA, Work current: 400 - 600mA |
|  | Working environment | Working temperature: - 10 ~ 55℃ , Working humidity <90% |
|  | Weight | (Note: in kind prevailed) |
|  | Size | 56×43×44mm, L*W*H |
|  | Packing list | Camera*1 , Bracket*1 , MICRO USB*1 , Manual*1 , SIM Card*1 |