Specifications Model：CB71

| System | Operation system | Embedded Linux OS |
| --- | --- | --- |
|  | Online visitor | Support 4 visitors at the same time |
| Image Capture | Image sensor | 1/2.9-inch 1080p Progressive Scan CMOS sensor |
|  | Minimum Illumination | 0.3-0.5Lux(color mode), 0 Lux(B&W mode) |
|  | Lens | 3.2mm module |
|  | View Angle | 145˚(Diagonal) |
|  | Night vision | Automatic lens sensing switch，6pcs 940nm SMD IR LED, IR irradiation distance:5-10 m |
| Video | Encode format | H.264 main profile/Motion-JPEG/JPEG |
|  | Streams | Main stream：1080p(1920×1080)@30fps Substream：360p(640×360)@15fps |
|  | Bit Rate/Max Fps | 128~4096kbps/30pfs |
|  | Image adjustment | Brightness and contrast can be adjusted |
|  | Encode format | H.264 main profile/Motion-JPEG/JPEG |
| Audio | Input | Built-in -38db microphone |
|  | Output | 8Ω0.5W speaker |
|  | Sampling frequency/Bit width | 8KHz/16bit |
|  | Compression standard /code rate | ADPCM/32kbps |
| Network | Ethernet | N/A (connect via AP hotpot mode) |
|  | Protocols | TCP/IP,HTTP,TCP,UDP,DHCP,DNS,NTP,P2P etc |
|  | Wireless LAN | IEEE802. 11b/g/n, Supports AP mode |
|  | Wireless LAN Frequency | 2.4~ 2.4835GHz |
|  | Wireless LAN security | 64/ 128-bit WEP/WPA-PSK/WPA2-PSK data encryption |
| Memory | Socket | TF Push-push socket,Maximum support 256GB ，cloud storage |
| Alarm | Alarm Trigger | PIR detection |
| Physical Environmental | Rated voltage | DC5V±5% |
|  | Power supply | Built-in 1500mAh batteries |
|  | Power consumption | Rated power: 2.05W (with infrared lights on) Maximum power: 3.0W(All function on)/Standby power:0.002W |
|  | Operating condition | Temperature:-10~50℃,humidity:＜90% |
|  | Weight | Net: 60g Gross：210g(Note: in kind prevail) |
|  | Package size | 133x104x60mm (L×W×H) |

Shenzhen VStarcam Technology Co., Ltd

www.vstarcam.com