| Sorts | Type | Parameters |
| --- | --- | --- |
| System | Operating system | Embedded Linux |
|  | Online visitor | Supports 4 visitors at the same time |
|  | CPU/Image sensor | 1/2.9 inch 1080p progressive scan CMOS sensor |
|  | Minimum illumination | 0.5 Lux(Color mode), 0.1 Lux(B&W mode) |
|  | Lens/Viewing angle | 3.6mm@F2.0/101°(Diagonal) |
|  | Night vision | IR-CUT with auto-switching，12 pcs LED lights, 18 pcs IR LED lights, Irradiation distance: 20~30 m |
|  | Compression standard | H.264 main profile/H.264+ main profile/Motion-JPEG/JPEG |
|  | Data Rate | Main stream: 1080p(1920×1080)@30fps/1296p(2304×1296)@15fps Substream: 360p(640×360)@15fps |
|  | Bit rate/Maximum frame rate | 128～4096kbps/30fps |
|  | Image adjustment | Brightness and contrast are adjustable |
| Audio | Input | Built-in -38dB microphone |
|  | Output | Built-in 4Ω3W Loud speaker 130 dB |
|  | Sampling frequency/bit width | 8KHz/16bit |
|  | Compression standard/Bit Rate | G.711/64kbps |
| Network | Network interface | 10Mbps/100Mbps adaptive /RJ45 |
|  | Network protocol | TCP/IP,HTTP,TCP,UDP,DHCP,DNS,NTP,RTSP,P2P etc. |
|  | Wireless network | IEEE802.11b/g/n；Support AP mode |
|  | Wireless frequency | 2.4~2.4835GHz |
|  | Wireless security encryption | 64/128-bit WEP/WPA-PSK/WPA2-PSK Data encryption |
| Pan & Tilt | Pan & Tilt | Pan 355° /Tilt 110° |
|  | Preset position | Supports 16 (Mobile phone App supports 5) preset positions |
| Storage | Storage functions | Supports T-Flash card(max supports 256GB); Cloud Storage; NVR；NAS |
| Alarm | Alarm detection | Motion detection; Humanoid detection; Humanoid tracking; Smoke sound detection; Absence detection; Alarm voice prompt , flash & irradiation; LED light timing ON & OFF |
| Physical Indicators | Rated voltage | DC12V±5% |
|  | Power consumption | Rated power: 4.5 W (With IR LED lights ON)/ Max power: 11 W (With Pan & Tilt ON) |
|  | Operating condition | Temperature: -10～50℃, Humidity: ＜90% |
|  | Weight | （Note: in kind prevail） |
|  | Package size | (230x205x140）mm |