| Sorts | Type | Parameters |
| --- | --- | --- |
| System | Operating system | Embedded Linux |
|  | Online visitor | Supports 4 visitors at the same time |
|  | CPU/Image sensor | 1/3’’ 4Mega CMOS Image Sensor |
|  | Minimum illumination | 0.5 Lux(Color mode), 0.1 Lux(B&W mode) |
|  | Lens/Viewing angle | 4mm@F2.0/101° Diagonal |
|  | Night vision | IR LED light+LED light; Irradiation distance: 15~20 m |
|  | Compression standard | H.264 main profile/H.264+ main profile/Motion-JPEG/JPEG |
|  | Data Rate | Main stream: 1440p(2560×1440）@25fps；Substream：360p(640×360)@15fps |
|  | Bit rate/maximum frame rate | 128～4096kbps/30fps |
|  | Image adjustment | Brightness and contrast are adjustable |
| Audio | Input | Built-in －38dB microphone |
|  | Output | Built-in 8Ω2 W speaker |
|  | Sampling frequency/bit width | 8KHz/16bit |
|  | Compression standard/Bit Rate | G.711/64kbps |
| Network | Network interface | 10Mbps/100Mbps adaptive /RJ45 |
|  | Network protocol | TCP/IP,HTTP,TCP,UDP,DHCP,DNS,NTP,RTSP,P2P etc. |
|  | Wireless network | IEEE802.11b/g/n；Supports AP mode |
|  | Wireless frequency | 2.4~2.4835GHz |
|  | Wireless security encryption | 64/128-bit WEP/WPA-PSK/WPA2-PSK dataencryption |
| Pan & Tilt | Pan & Tilt | Pan 345° /Tilt 53° |
|  | Preset position | Supports 16 (Mobile phone App supports 5) preset positions |
| Storage | Storage functions | Supports T-Flash card(max supports 256GB); Cloud Storage； NVR；NAS |
| Alarm | Alarm detection | Supports Motion detection; Humanoid detection; Humanoid tracking; Smoke sound detection; Absence detection; LED light Timing ON & OFF |
| Physical Indicators | Rated voltage | DC12V±5% |
|  | Power consumption | Rated power：5W (with IR LED lights ON)/Max power：11W (with Pan & Tilt ON) |
|  | Operating condition | Temperature: -10～50℃; Humidity: ＜90% |
|  | Weight | Net: 472g（Note: in kind prevail） |