| Sorts | Type | Parameters |
| --- | --- | --- |
| System System | Operating system | Embedded Linux |
|  | Online visitor | Supports 4 visitors at the same time |
| Collection | CPU/Image sensor | 1/2.9 inch progressive scan CMOS sensor |
|  | Minimum illumination | 0.5 Lux（Color mode）, 0.1 Lux（B&W mode） |
|  | Lens/Viewing angle | 3.5-9.5mm zoom lens / 41～110° Diagonal |
|  | Night vision | IR-CUT with auto-switching，4 pcs 850nm IR LED lights, 4 high-power LED lights; Irradiation distance: 15~20 m |
| Video | Compression standard | H.264 main profile/H.264+ main profile/Motion-JPEG/JPEG |
|  | Data rate | Main stream: 1296p@15fps(2304×1296)/1080p@30fps(1920×1080) Substream：360p@15fps(640×360) |
|  | Bit rate/maximum frame rate | 128～4096kbps/30fps |
|  | Image adjustment | Brightness and contrast are adjustable |
| Audio | Input | Built-in -28dB microphone |
|  | Output | Built-in 8Ω2W speaker |
|  | Sampling frequency/bit width | 8KHz/16bit |
|  | Compression standard/Bit Rate | G.711/32kbps |
| Network | Network interface | 10Mbps/100Mbps adaptive /RJ45 interface |
|  | Network protocol | TCP/IP,HTTP,TCP,UDP,DHCP,DNS,NTP,RTSP,P2P，ONVIF etc. |
|  | Wireless network | IEEE802.11b/g/n; Supports AP mode |
|  | Wireless frequency | 2.4GHz |
|  | Wireless security encryption | 64/128-bit WEP/WPA-PSK/WPA2-PSK data encryption |
| Storage | Storage functions | Supports T-Flash card(max supports 256GB); Cloud Storage；NVR；NAS |
| Alarm | Alarm detection | Supports Motion detection; Humanoid detection; Humanoid framing; Smoke sound detection; Absence detection; Delimitation detection |
| Physical Indicators | Rated voltage | DC12V±5% |
|  | Operating condition | Temperature: -10～50℃; Humidity: ＜90% |
|  | Weight | （Note: in kind prevail） |