| Sorts | Type | Parameters |
| --- | --- | --- |
| System | Operating system | Embedded Linux |
|  | Online visitor | Supports 4 visitors at the same time |
| Collection | Image sensor | 1/3"CMOS |
|  | Lens | 4.7-141mm zoom lens |
|  | Aperture | F1.5-F4.0 |
|  | Minimum illumination | 0.1 Lux（B&W mode），0.5 Lux（Color mode） |
|  | Viewing angle | D：64.2°@1X，3.9°@30X |
|  | Night vision | IR-CUT with auto-switching，6 pcs 850nm IR LED lights+3 pcs laser lights; Irradiation distance: 100~200 m |
| Video | Compression standard | H.264 main profile/H.264+ main profile/Motion-JPEG/JPEG |
|  | Multi-stream | Main stream: 1520P（2688*1520）@15fps/1440p(2560×1440)@30fps Substream: 360p(640×360)@15fps |
|  | Bit rate | 128～4096kbps |
|  | Maximum frame rate | 30fps |
|  | Image adjustment | Brightness and contrast are adjustable |
| Audio | Input | Built-in －28dB microphone |
|  | Output | Built-in 8Ω2 W speaker |
|  | Sampling frequency/bit width | 8KHz/16bit |
|  | Compression standard/Bit Rate | G.711/32kbps |
| Network | Network interface | 10Mbps/100Mbps adaptive /RJ45 |
|  | Network protocol | TCP/IP,HTTP,TCP,UDP,DHCP,DNS,NTP,RTSP,P2P，ONVIF etc. |
|  | Wireless network | IEEE802.11b/g/n；AP |
|  | Wireless frequency | 2.4G( WiFi6 ) |
|  | Wireless security encryption | 64/128-bit WEP/WPA-PSK/WPA2-PSK data encryption |
| Pan & Tilt | Pan & Tilt | Pan 360°/Tilt 98° |
|  | Preset position | Supports 16(Mobile phone App supports 5) preset positions |
| Storage | Storage functions | Supports T-Flash card(max supports 256GB); Cloud storage; NVR；NAS |
| Alarm | Alarm detection | Supports Motion detection; Humanoid detection; Humanoid framing; Siren alarm; Absence detection; Delimitation detection |
| Physical Indicators | Rated voltage | DC12V or POE（48V） |
|  | Power consumption | Standby power: 2.2W, Rated power：12.8 W(with IR LED lights ON)/Max power：22 W |
|  | Waterproof standard | IP65 |
|  | Operating condition | Temperature: -10～50℃; Humidity: ＜90% |
|  | Weight | Net：2400g （Note: in kind prevail） |
|  | Size | 420x320x235mm（LxWxH） |