# NVR(N8209)specification

| MODEL |  | N8209 |
| --- | --- | --- |
| Video Input | IP Video Input | 9 CH 720P/960P/1080P，RJ45 interface |
|  | Switch | NO |
| Video Encoding | HDMI Output | 1 CH ，HDMI output ；Resolution：1920x1080P/60Hz; |
|  | VGA Output | 1 CH ，VGA output ；Resolution：1920x1080P/60Hz |
|  | Audio Output | HDMI Audio output, optional for 3.5mm audio port output |
| Video Encoding | Video Compression | stander H.265, H.264 |
|  | Recording /Playback | 9CH 1080P record, 2 CH 1080P playback |
|  | Video Resolution | 720P，960P，1080P record |
|  | Frame Rate | PAL: 25 F/S；NTSC :30 F/S |
|  | Video Bitrate | 256 Kbps -- 4.0Mbps(@1080P) |
|  | Stream Type | Audio/Video,Variable Rate(VBR) , Constant Bit Rate(CBR) |
|  | Dul-Stream | supprt |
|  | Audio Compression | G.711 |
|  | Stream | 70Mbps |
| HDD | Storage Type | 1 * SATA |
|  | Storage Capacity | 6TB In Max |
|  | Backup | USB Backup,remote backup |
| External Interface | Network Interface | RJ45 10M / 100M Internet Interface |
|  | USB Interface | 2 * USB2.0 Interface(Support Upgrade,Backup,Mouse) |
|  | Serial Interface | 1 * RS485 Interface |
| Others | Temperature | -10°---- 55° |
|  | Size | TBD |
|  | Weight | TBD |
|  | Operation System | Embedded Linux Operation System |
|  | Soft Power-Off | Not Support |
|  | Device Control | IR remote control,Mouse |
|  | Power Input | DC +12 V / 2A |